Protocol of AhR HepG2 Cell-based Assay for High-throughput Screening

| DOCUMENT: |  | AhR_TOX21_SLP_Version1.0 |
| --- | --- | --- |
| TITLE: |  | Protocol of AhR HepG2 Cell-based Assay for High-throughput Screening |

ASSAY RFERENCES:

| Assay Target | Cell Lines | Species | Tissue of Origin | Assay Readout | Assay Provider | Toxicity Pathway |
| --- | --- | --- | --- | --- | --- | --- |
| Human AhR (Recombinant) | HepG2 | Human | Hepatocellular carcinoma | Luciferase reporter | Denison Lab | NR signaling |

QUALITY CONTROL PRECAUTIONS:

MATERIALS and INSTRUMENTS:

| Supplies/Medium/Reagent | Manufacturer | Vender/Catalog Number |
| --- | --- | --- |
| -Alpha-MEM | -Invitrogen | -12561 |
| -Premium FBS | -Atlanta Biologicals | -S11150 |
| -G418 | -Invitrogen | -10131-027. |
| -0.25% trypsin/EDTA | -Invitrogen | -25300 |
| -PBS | -Invitrogen | -14190 |
| -DMSO | -AMRESCO | -KD Medical, RGE-3070 |
| -Recovery cell culture freezing medium | -Invitrogen | -12648 |
| -White solid bottom plates | -Greiner | - |
| -omeprazole | -Sigma Aldrich | - |
| -Pintool | -Kalypsis | - |
| -Viewlux | -PerkinElmer | - |
| -Centrifuge | -Sorvall legend XTR | -Thermo Fisher Science 75004520 |
| -tetraoctylammonium bromide | -sigma | -88000 |
| -One-Glo | -Promega | -E6130 |
| -celltiter fluor | -Promega | -G6082 |

PROCEDURE:

1. Cell handling:

1.1. Media Required:

| Component | Growth Medium | Assay Medium | Thaw Medium | Freezing Medium |
| --- | --- | --- | --- | --- |
| -Alpha MEM | -90% | -90% | -90% | - |
| -Premium FBS | -10% | -10% | -10% | - |
| -G418 | -400mg/mL | - | - | - |
| -Recovery cell culture freezing medium | - | - | - | -100% |

1.2. Thawing method

1.2.1 -place 14mL of pre-warmed thaw media into T75 flask

1.2.2 -remove vial of cells to be thawed from -140 and thaw rapidly by placing in water bath with gentle agitation for 1-2min

1.2.3 -Wipe vial with 70% ethanol before opening in biological safety cabinet

1.2.4 -transfer vial contents dropwise into 10mL of thaw medium in 15mL conical tube

1.2.5 -centrifuge cells at 1000 rpm for 4min

1.2.6 -Resuspend and transfer contents into T75 flask containing thaw medium and transfer flask into incubator

1.2.7 -switch to growth media at first passage

1.3. Propagation method

1.3.1 -aspirate media, rinse once with dPBS, add 0.25% trypsin/EDTA and swirl to coat flask evenly

1.3.2 -add equal volume of growth medium to inactivate trypsin after 2-3 minutes incubation

1.3.3 -centrifuge cells at 1000 RPM for 4min and resuspend in growth medium before adding to new flask

1.3.4 -cells should be passaged or fed at least twice per week

2. Assay Protocol

2.1 -Harvest cells from culture in growth medium and resuspend in assay medium

2.2 -dispense 4000 cells per well into 1536-well tissue culture treated white solid bottom plate using multidrop dispenser

2.3 -Incubate cells 5hr, then dispense 23nL of compound, positive control, or DMSO control using pintool

2.4 -positive and control compounds are located in the first four columns according to the plate map, and library compounds located in columns 5-48

2.5 -incubate plates for 19hr at 37 degrees C

2.6 -add 1uL of celltiter fluor to each well using bioraptr dispenser

2.7 -incubate 30min at 37 degrees C

2.8 -measure fluorescence using ViewLux

2.9 -Add 5uL of OneGlo using bioraptr dispenser

2.10 -incubate 30min at room temperature

2.11 -Read luminescence on ViewLux

3. Assay Performance

| AhR (Omeprazole) | Online Validation Agonist (Mean ± SD) | Online Validation Viability (Mean ± SD) |
| --- | --- | --- |
| EC50 | 33.2 ± 26.0 μM (n = 27) | NA |
| S/B | 4.83 ± 0.55 | 5.03 ± 0.16 |
| CV (%) ⃰ | 16.64 ± 4.29 (n = 18) | 7.76 ± 0.54 (n = 18) |
| Z’ | 0.35 ± 0.09 | 0.69 ± 0.03 |

* CV values shown represent average of DMSO plates and low concentration plates only.