Protocol of AR-MDA MDA-MB-453 Cell-based Assay for High-throughput Screening

| DOCUMENT: |  | AR-MDA_TOX21_SLP_Version1.0 |
| --- | --- | --- |
| TITLE: |  | Protocol of AR-MDA MDA-MB-453 Cell-based Assay for High-throughput Screening |

ASSAY RFERENCES:

| Assay Target | Cell Lines | Species | Tissue of Origin | Assay Readout | Assay Provider | Toxicity Pathway |
| --- | --- | --- | --- | --- | --- | --- |
| Androgen receptor | MDA-MB-453 | Human | Mammary gland, breast | Luminescence | ATCC | NR signaling |

QUALITY CONTROL PRECAUTIONS:

1. -Maintain cell culture below 85-90% confluence

2. -Cell culturing and assay culture doesn’t require CO2

MATERIALS and INSTRUMENTS:

| Supplies/Medium/Reagent | Manufacturer | Vender/Catalog Number |
| --- | --- | --- |
| -Leibovitz's L-15 Medium | -ATCC | -ATCC / 30-2008 |
| -Fetal Bovine Serum | -Hyclone | -Hyclone / SH30071.03 |
| -Penicillin/Streptomycin | -Invitrogen | -Invitrogen / 15140 |
| -Recovery Cell culture Freezing Medium | -Invitrogen | -Invitrogen / 12648 |
| -0.05% Trypsin-EDTA | -Invitrogen | -Invitrogen / 25300 |
| -1536-well white solid plates | -Corning | -Corning / 7464 |
| -MULTIDROP COMBI | -Thermo Electron Corporation | -Thermo Electron Corporation |
| -BioRAPTR FRD | -Beckman Coulter | -Beckman Coulter |
| -ViewLux Plate Reader | -Perkin Elmer | -Perkin Elmer |
| -ONE-Glo(TM) Luciferase Assay System | -Promega | -Promega/E6120 |
| -CellTiter-Fluor(TM) Cell Viability Assay | -Promega | -Promega/G6082 |
| -R1881 (Agonist positive control compound) | -Perkin Elmer | -Perkin Elmer/NLP005005MG |
| -Nilutamide (Antagonist positive control compound) | -Sigma Aldrich | -Sigma Aldrich/N8534 |
| -Tetraoctyl ammonium bromide (Viability positive control compound) | -Sigma Aldrich | -Sigma Aldrich/294136 |

PROCEDURE:

1. Cell handling:

1.1. Media Required:

| Component | Growth Medium | Assay Medium | Thaw Medium | Freezing Medium |
| --- | --- | --- | --- | --- |
| -Leibovitz's L-15 Medium | -100% | -100% | -100% | - |
| -Fetal Bovine Serum | -10% | -10% | -10% | - |
| -Penicillin & Streptomycin | -100U/ml & 100ug/ml | -100U/ml & 100ug/ml | -100U/ml & 100ug/ml | - |
| -Recovery Cell culture Freezing Medium | - | - | - | -100% |

1.2. Thawing method

1.2.1 -Thaw a vial of cells in 9ml of pre-warmed thaw/culture medium and then centrifuge

1.2.2 –Re-suspend the pellet with the thaw/culture medium and seed at 2 million cells per T-75 flask

1.3. Propagation method

1.3.1 -Trypsinize cells from the culturing flask and centrifuge and then re-suspend cells in culture medium

1.3.2 -Passage cells at 6-7 million per T-225 flask

2. Assay Protocol

2.1 -Trypsinize cells from the culturing flask and centrifuge and then re-suspend cells in culture/assay medium.

2.2 -Dispense 3000 cells/4uL/well into 1536-well tissue treated white/solid bottom plates using an 8 tip dispenser (Multidrop).

2.3 -Incubate the assay plates for 5hrs at 37C and 0% CO2.

2.4 -First 1uL of 3.0uM (final concentration) Nilutamide or assay buffer was added using two separate tips of a dispenser (BioRAPTR).

2.5 -Then 23nL of compounds were transferred from the library collection into 5-48 columns and positive control into 1-4 columns using a Pintool station.

2.6 -Incubate the assay plates for 16hrs at 37C and 0% CO2.

2.7 -After 15hrs of incubation, 1ul of CellTiter-Fluor(TM) Cell Viability Assay reagent was added using a single tip of a dispenser (BioRAPTR).

2.8 -Incubate the assay plates at 37C and 0% CO2 for 1hr.

2.9 -Measure fluorescence signal by ViewLux plate reader (Exposure time = 1sec).

2.10 -Add 4ul of ONE-Glo(TM) Luciferase reagent using a single tip of a dispenser (BioRAPTR).

2.11 -Incubate the assay plates at room temperature for 30min.

2.12 -Measure luminescence signal by ViewLux plate reader (Exposure time = 90sec).

3. Assay Performance

| AR-MDA-kb2 Nilutamide added | Online Validation Agonist (Mean ± SD) | Online Validation Viability (Mean ± SD) |
| --- | --- | --- |
| EC50 Nilutamide-free R1881 | 0.29 ± 0.04 nM (n = 27) | NA |
| EC50 R1881 with 3.0uM Nilutamide | 8.16 ± 1.05 nM (n = 26) | NA |
| S/B | 9.07 ± 0.37 | 10.23 ± 0.25 |
| CV (%) ⃰ | 7.57 ± 0.89 (n = 24) | 6.21 ± 0.38 (n = 24) |
| Z’ | 0.76 ± 0.05 | 0.81 ± 0.02 |

⃰ CV values shown represent average of all assay plates excluding the top concentration plates