Protocol of CHRM1-CHO Cell-based Assay for High-throughput Screening

DOCUMENT:		CHRM1-CHO _TOX21_SLP

TITLE:	Protocol of CHRM1-CHO cell-based assay for high-throughput screening.

ASSAY RFERENCES:

| Assay Target | Cell Lines | Species | Tissue of Origin | Assay Readout | Assay Provider | Toxicity Pathway |
| --- | --- | --- | --- | --- | --- | --- |
| muscarinic acetylcholine receptor M1 (CHRM1) | CHO | Hamster | Ovary | Fluorescence | Codex | Gq-coupled CHRM1 signaling |

QAlity control PRECAUTIONS:

- Cells should be grown to reach 80 to 90% confluence.

MATERIALS and INSTRUMENTS:

| ### Supplies/Medium/Reagent | Vender | Catalog Number |
| --- | --- | --- |
| ### DMEM/F12 | Sigma | D8437 |
| ### Fetal bovine serum | Invitrogen | 26140 |
| ### Penicillin/Streptomycin | Invitrogen | 15140 |
| ### DPBS | Invitrogen | 14190 |
| ### 0.05% Trypsin/EDTA | Invitrogen | 25300 |
| ### Puromycin | Invitrogen | A11138 |
| ### Recovery™ Cell Culture Freezing Medium | Invitrogen | 12648 |
| ### Acetylcholine | Sigma |  |
| ### Atropine | Sigma |  |
| ### Cal-520 calcium assay kit | AAT Bioquest |  |
| ### Black-clear bottom, 1536-well assay plates | Greiner Bio-One |  |
| ### BioRAPTR™, Microfluidic Workstation | Beckmen |  |
| ### FDSS 7000EX kinetic plate reader | Hamamatsu |  |

PROCEDURE:

- Cell handling:

  - Medium Required

| Component | Growth Medium | Assay Medium | Thaw Medium | Freezing Medium |
| --- | --- | --- | --- | --- |
| DMEM | 90% | 90% | 90% | - |
| FBS | 10% | 10% | 10% | - |
| Penicillin-Streptomycin | 100 U/mL-100 μg/mL | 100 U/mL-100 μg/mL | 100 U/mL-100 μg/mL | - |
| Puromycin | 10 μg/mL | - | - | - |
| ### Recovery™ Cell Culture Freezing Medium | - | - | - | 100% |

  - Thawing method

- Place 14 mL of pre-warmed thaw medium into a 15 mL of conical tub.

- Remove the vial of cells to be thawed from liquid nitrogen and thaw rapidly by placing at 37°C in a water bath with gentle agitation for 1-2 minutes.

- Decontaminate the vial by wiping with 70% ethanol before opening in a biological safety cabinet.

- Transfer the vial contents drop-wise into 14 mL of thaw medium in a sterile 15-mL conical tub.

- Centrifuge cells at 900 rpm for 4 minutes and resuspend in thaw medium.

- Transfer contents to the T75 tissue culture flask containing Thaw Medium and place flask in a humidified 37°C/5% CO2 incubator.

- Switch to growth medium at first passage.

  - Propagation method

- Aspirate medium, rinse once in DPBS, add 0.05% Trypsin/EDTA and swirl to coat the cell evenly.

- Add an equal volume of Growth Medium to inactivate Trypsin after 2-3 minutes incubation at 37°C.

- Centrifuge cells at 900 rpm for 4 minutes and resuspend in growth medium.

- Cell should be passage or fed at least twice a week.

- Assay protocol for antagonist mode

- Harvest cells and resuspend in assay medium.

- Dispense 1000cells/3µL/well into 1536-well tissue treated black-clear-bottom plates using a Multi-drop dispenser.

- After the cells were incubated at 37°C for 18 hours, add 3µl of Loading Dye to each well.

- Incubate the plates for 1 h at 37°C.

- Incubate the plates for 15 m at room temperature

- Add 23 nL of compounds dissolved in DMSO, positive controls or DMSO by a PinTool.

- Read the fluorescence intensity in FDSS 7000EX kinetic plate reader with a filter set of Ex/Em=480/540 for 2 min at 1 sec intervals

- For antagonist mode, incubate the plates for 5 m at room temperature after step 2.6

- Add 1.5 µL of Acetylcholine at 50 nM to each well

- Read the fluorescence intensity in FDSS 7000EX kinetic plate reader with a filter set of Ex/Em=480/540 for 3 min at 1 sec intervals

3. Assay Performance

| CHRM-CHO agonist | Online Validation (Mean ± SD) |
| --- | --- |
| EC50 (nM) | 17.23 ± 3.70 |
| S/B | 56.45 ± 17.32 |
| CV (%) | 19.27 ± 6.51 |
| Z’ | 0.83 ± 0.03 |

| CHRM-CHO antagonist | Online Validation (Mean ± SD) |
| --- | --- |
| EC50 (nM) | 3.10 ± 0.75 |
| S/B | 36.89 ± 7.36 |
| CV (%) | 7.06 ± 2.27 |
| Z’ | 0.80 ± 0.06 |