Protocol of CRE-bla HEK 293T Cell-based Assay for High-throughput Screening (Agonist mode)

| DOCUMENT: |  | CRE BLA_TOX21_SLP_Version1.0 |
| --- | --- | --- |
| TITLE: |  | Protocol of CRE-bla HEK 293 Cell-based Assay for High-throughput Screening |

ASSAY RFERENCES:

| Assay Target | Cell Lines | Species | Tissue of Origin | Assay Readout | Assay Provider | Toxicity Pathway |
| --- | --- | --- | --- | --- | --- | --- |
| CRE LBD (Recombinant) | HEK 293 | Human | Embryonic kidney cells | Beta-lactamase reporter | Invitrogen | NR signaling |

QUALITY CONTROL PRECAUTIONS:

1. The cells should not be grown more than 80-85% confluence

2. The cell performance is affected if they are too confluent

3. Do not leave cells in Trypsin for more than 5 min at RT

4. Handle 1536 well plate black clear bottom plates carefully by sides

MATERIALS and INSTRUMENTS:

| Supplies/Medium/Reagent | Manufacturer | Vender/Catalog Number |
| --- | --- | --- |
| DMEM | Invitrogen | 11965 |
| Dialyzed FBS | Invitrogen | 26400 |
| Sodium Pyruvate | Invitrogen | 11360 |
| Pen-Strep | Invitrogen | 15140 |
| NEAA | Invitrogen | 11140 |
| HEPES | Invitrogen | 15630 |
| Blasticidin | Invitrogen | R21001 |
| 0.05% Trypsin-EDTA | Invitrogen | 25300-054 |
| Forskolin (NKH477) | Sigma | 344281 |
| Multidrop | Thermofisher | - |
| BiorapTR | Beckman Coulter | - |
| Envision Plate Reader | Perkin Elmer | - |
| LiveBLAzer B/G FRET substrate | Invitrogen | K1030 |

PROCEDURE:

1. Cell handling:

1.1. Media Required:

| Component | Growth Medium | Assay Medium | Thaw Medium | Freezing Medium |
| --- | --- | --- | --- | --- |
| Recovery Cell Freezing Medium | - | - | - | 100% |
| DMEM | 90% | 90% | 90% | - |
| Dialyzed FBS | 10% | 10% | 10% | - |
| Sodium Pyruvate | 1mM | 1mM | - | - |
| Penn-strep | 1% | 1% | 1% | - |
| HEPES | 25 mM | 25 mM | 25 mM | - |
| NEAA | 0.1 mM | 0.1 mM | 0.1 mM | - |
| Blasticidin | 5 ug/ml | - | - | - |

1.2. Thawing method

1.2.1 -Place 14 ml of pre-warmed thaw medium into a 15 ml conical tube

1.2.2 -Remove the vial of cells to be thawed from liquid nitrogen and thaw rapidly by placing at 37°C in a water bath with gentle agitation for 1-2 minutes. Do not submerge vial in water.

1.2.3 -Mix the entire content of the vial to 14 ml of pre-warmed medium and centrifuge to remove DMSO

1.2.4 -Discard the supernatant and transfer the precipitated cells to T225 flask using 45 ml thawing medium or five-layer flask using 200ml thawing medium

1.3. Propagation method

1.3.1 -Detach the cells from the flask using 0.05% Trypsin-EDTA

1.3.2 -The cells are re-seeded in T-225 flask at 3 million or in five-layer flask at 15 million

2. Assay Protocol

2.1 -Spin down the cells after rinsing the cells with DPBS and trypsin

2.2 -Resuspend the pellet with assay medium followed by filtering through cell strainer and adjust to the required cell density

2.3 -Plate the cells in black-clear bottom 1536 well plate at 2000/well/5µl for agonist mode through 8 tip Multidrop plate dispenser

2.4 -Incubate for 18hrs at 37ºC / 95% Humidity / 5% CO2

2.5 -Transfer 23nL of compounds from the library collection and positive control to the assay plates through pintool

2.6 -Incubate for 3hrs at 37ºC / 95% Humidity / 5% CO2

2.7 -Add 1µl of CCF4 (FRET Substarte) dye using a single tip plate dispenser (Bioraptr)

2.8 -Incubate at room temperature for 2hrs in dark

2.9 -Read the fluorescence intensity through Envision plate reader using Beta-Lactamaze protocol optimized for this cell type

2.10 -Add 4µl of CellTiter Glo and Incubate at room temperature for 0.5hrs in dark

2.11 -Read on ViewLux protocol optimized for this cell type

3. Assay Performance

3.1 Performance of online validation:

| CRE-bla HEK293 Agonist (NKH477) | Online Validation (Mean ± SD) | Online Validation Viability (Mean ± SD) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EC50 | 0.27 ± 0.06 µM (n =27) | N/A |  |  |  |  |  |
| S/B | 4.16 ± 0.24 | 126.63 ± 6.13 |  |  |  |  |  |
| CV (%) | 6.59 ± 0.97 (n = 24) | 4.71 ± 1.01 (n = 24) |  |  |  |  |  |
| Z’ | 0.82 ± 0.02 | 0.66 ± 0.04 |  |  |  |  |  |

* CV values shown represent average of all assay plates excluding the top concentration plates.

3.2 Performance of online screening:

| Assay | Active match | Inactive match | Inconclusive | Mismatch | AC50 fold change | Score |
| --- | --- | --- | --- | --- | --- | --- |
| tox21-cre-agonist-p1_ratio | 1.01% | 98.20% | 0.79% | 0.00% | 1.32 | 99.44 |
| tox21-cre-agonist-p1_viability | 2.66% | 93.35% | 3.97% | 0.02% | 1.41 | 94.65 |