Protocol of GR-BLA HeLa Cell-based Assay for High-throughput Screening

| DOCUMENT: |  | GR-BLA_TOX21_SLP_Version1.0 |
| --- | --- | --- |
| TITLE: |  | Protocol of GR-BLA HeLa Cell-based Assay for High-throughput Screening |

ASSAY RFERENCES:

| Assay Target | Cell Lines | Species | Tissue of Origin | Assay Readout | Assay Provider | Toxicity Pathway |
| --- | --- | --- | --- | --- | --- | --- |
| Glucocorticoid receptor (Endogenous) | HeLa | Human | Cervical carcinoma | Beta-lactamase reporter | Invitrogen | NR signaling |

QUALITY CONTROL PRECAUTIONS:

1. -cells should not be grown more than 80-85% confluence

2. -The cell performance is affected is they are more confluent

3. -handle 1536 well plate black clear bottom plates carefully by sides

MATERIALS and INSTRUMENTS:

| Supplies/Medium/Reagent | Manufacturer | Vender/Catalog Number |
| --- | --- | --- |
| -DMEM with Glutamax | -Invitrogen | -10566 |
| -Dialyzed FBS | Invitrogen | -26400 |
| NEAA | -Invitrogen | -11140 |
| -HEPES | -Invitrogen | -15630 |
| -Sodium Pyruvate | -Invitrogen | -11360 |
| -Pennicillin/Streptomycin | -Invitrogen | -15140 |
| -Blasticidin | -Invitrogen | -R21001 |
| -multidrop | -Thermofisher | - |
| -BiorapTR dispenser | -Beckman Coulter | -Beckman Coulter |
| -Envision plate reader | -Perkin Elmer | -Perkin Elmer |
| -LiveBLAzer B/G FRET substrate | -Invitrogen | -K1030 |
| -Opti MEM | -Invitrogen | -11058 |

PROCEDURE:

1. Cell handling:

1.1. Media Required:

| Component | Growth Medium | Assay Medium | Thaw Medium | Freezing Medium |
| --- | --- | --- | --- | --- |
| -Recovery Cell freezing medium | - | - | - | -100% |
| -DMEM with Glutamax | -90 % | - | -90 % | - |
| -Dialyzed FBS | -10 % | -1% | -10 % | - |
| -NEAA | -0.1 mM | -0.1 mM | -0.1 mM | - |
| -HEPES | -25 mM | -25 mM | -25 mM | - |
| -Na-Pyruvate | -1 mM | -1 mM | -1 mM | - |
| -Penn/Strep | -1% | -1% | -1% | - |
| -Blasticidin | -5 ug/ml | - | - | - |
| -OptiMEM | - | -99% | - | - |

1.2. Thawing method

1.2.1 -Place 14 mL of pre-warmed thaw medium into a 15 ml conical tube

1.2.2 -Remove the vial of cells to be thawed from liquid nitrogen and thaw rapidly by placing at 37°C in a water bath with gentle agitation for 1-2 minutes. Do not submerge vial in water.

1.2.3 -Mix the entire content of the vial to 14 ml of pre-warmed medium and centrifuge to remove DMSO

1.2.4 -Discard the supernatent and transfer the precipitated cells to T175 flask using 30 ml thawing medium

1.3. Propagation method

1.3.1 -Detach the cells from the flask using TrypLExpress

1.3.2 -The cells are re-seeded in T-175 flask at 1-1.3 million

2. Assay Protocol

2.1 -Spin down the cells after rinsing the cells with DPBS and trypsinizing

2.2 -Resuspend the pellet with assay medium

2.3 -Plate the cells in black-clear bottom 1536 well plate at 1500/well/6uL through 8 tip Multidrop plate dispenser

2.4 -Incubate for 4 hrs at 37ºC / 99% Humidity / 5% CO2

2.5 -Transfer 23nL of compounds from the library collection and positive control to the assay plates through pintool

2.6 -Incubate for 18 hrs at 37ºC / 99% Humidity / 5% CO2

2.7 -Add 1uL of CCF4 (FRET Substarte) dye using a single tip plate dispenser (Bioraptr)

2.8 -Incubate at room temperature for 2 hrs in dark

2.9 -Read the fluorescence intensity through Envision plate reader using Beta-Lactamaze protocol optimized for this cell type

3. Assay Performance

| GR-bla Agonist (Dexamethasone) | Online Validation (Mean ± SD) |
| --- | --- |
| EC50 | 3.60 ± 0.78 nM (n = 27) |
| S/B | 2.95 ± 0.70 |
| CV (%) | 3.54 ± 0.40* (n = 21) |
| Z’ | 0.64 ± 0.06 |

* CV values shown represent average of DMSO plates and low concentration plates only.