Protocol of HSE-BLA HeLa Cell-based Assay for High-throughput Screening

| DOCUMENT: |  | HSE-BLA_TOX21_SLP_Version1.0 |
| --- | --- | --- |
| TITLE: |  | Protocol of HSE-BLA HeLa Cell-based Assay for High-throughput Screening |

ASSAY RFERENCES:

| Assay Target | Cell Lines | Species | Tissue of Origin | Assay Readout | Assay Provider | Toxicity Pathway |
| --- | --- | --- | --- | --- | --- | --- |
| Hsp70 | HeLa | Human | Cervical carcinoma | FRET | Invitrogen | Stress response |

QUALITY CONTROL PRECAUTIONS:

MATERIALS and INSTRUMENTS:

| Supplies/Medium/Reagent | Manufacturer | Vender/Catalog Number |
| --- | --- | --- |
| -Fetal bovine serum, dialyzed | -Invitrogen | -Invitrogen, 26400 |
| -Nonessential amino acids (NEAA) | -Invitrogen | -Invitrogen, 11140 |
| -Penicillin/Streptomycin (antibiotic) | -Invitrogen | -Invitrogen, 15140 |
| -Sodium Pyruvate | -Invitrogen | -Invitrogen, 11360 |
| -CCF4 dye | -Invitrogen | -Invitrogen, K1030 |
| -DMEM | -Invitrogen | -Invitrogen, 10564 |
| -Recovery Cell Freezing Media | -Invitrogen | -Invitrogen, 12648 |
| -Blasticidin | -Invitrogen | -Invitrogen, A11139-03 |
| -OptiMEM | -Invitrogen | -Invitrogen, 11058 |

PROCEDURE:

1. Cell handling:

1.1. Media Required:

| Component | Growth Medium | Assay Medium | Thaw Medium | Freezing Medium |
| --- | --- | --- | --- | --- |
| -DMEM | -90% | - | -90% | - |
| -Dialyzed FBS | -10% | -0.5% | -10% | - |
| -1 mM Sodium Pyruvate | -1% | -1% | -1% | - |
| -0.1 mM NEAA | -1% | -1% | -1% | - |
| -100 U/ml Penn-Strep | -1% | -1% | -1% | - |
| -Blasticidin | -500 ul | - | - | - |
| -OptiMEM | - | -99.5 | - | - |
| -Recovery Cell Freezing Media | - | - | - | -100% |

1.2. Thawing method

1.3. Propagation method

2. Assay Protocol

2.1 -1. Plate the cells at 1.5k/6uL/well in black-clear bottom 1536 well plates with assay medium.

2.2 -2. Incubate for an overnight at 37ÂºC / 99% Humidity / 5% CO2.

2.3 -3. Pin the compounds.

2.4 -4. Incubate at 37ÂºC / 99% Humidity / 5% CO2 for 5- 6hrs.

2.5 -5. Add 1uL of CCF4 dye (Solution A+B+C+D) and incubate for 2-2.5 hrs at RT and read the fluorescence through Envision. Ex 405 nm Em 460 and 530 nm.

2.6 -6. While running the plate on Envision, please run optimization for gain in advanced mode using one well with max signal for quick optimization.

3. Assay Performance

| HSP-bla Agonist (17AAG) | Online Validation (Mean ± SD) |
| --- | --- |
| EC50 | 0.08 ± 0.01 μM (n = 27) |
| S/B | 2.94 ± 0.17 |
| CV (%) | 4.02 ±0.40 * (n = 20) |
| Z’ | 0.78 ± 0.01 |

* CV values shown represent average of DMSO plates and low concentration plates only.