Protocol of HEK293 WT Cell-based Assay for High-throughput Screening

| DOCUMENT: |  | HEK293_WT_TOX21_SLP |
| --- | --- | --- |
| TITLE: |  | Protocol of WT HEK293 Cell-based Ca2+ Assay for High-throughput Screening |

ASSAY RFERENCES:

| Assay Target | Cell Lines | Species | Tissue of Origin | Assay Readout | Assay Provider | Toxicity Pathway |
| --- | --- | --- | --- | --- | --- | --- |
| Non-specific Ca++ signaling | HEK293 | Human | Embryonic kidney | Calcium dye | Codex Biosolutions | GPCR/Ca++ signaling |

QUALITY CONTROL PRECAUTIONS:

1. Maintain cells below 70-80% confluence

MATERIALS and INSTRUMENTS:

| Supplies/Medium/Reagent | Manufacturer | Vender/Catalog Number |
| --- | --- | --- |
| DMEM | Gibco | Gibco/ 11995-065 |
| Fetal Bovine Serum | Hyclone | Hyclone / SH30071.03 |
| Penicillin/Streptomycin | Invitrogen | Invitrogen / 15140 |
| Recovery Cell culture Freezing Medium | Invitrogen | Invitrogen / 12648 |
| 0.05% Trypsin-EDTA | Invitrogen | Invitrogen / 25300 |
| 1536-well black clear plates | Greiner Bio-One | Greiner Bio-One / 789092-F |
| MULTIDROP COMBI | Thermo Electron Corporation | Thermo Electron Corporation |
| BioRAPTR FRD | Beckman Coulter | Beckman Coulter |
| FDSS 7000EX kinetic plate reader | Hamamatsu | Hamamatsu |
| Screen Quest™ Calbryte-520 Probenecid-Free and Wash-Free Calcium Assay Kit | AAT Bioquest | AAT Bioquest / 36319 |
| ATP (Synonyms: Adenosine 5'-triphosphate) | MedChemExpress | MedChemExpress / HY-B2176 |

PROCEDURE:

1. Cell handling:

1.1. Media Required:

| Component | Growth Medium | Assay Medium | Thaw Medium | Freezing Medium |
| --- | --- | --- | --- | --- |
| DMEM | 90% | 90% | 90% | - |
| Fetal Bovine Serum | 10% | 10% | 10% | - |
| Penicillin & Streptomycin | 100U/ml & 100ug/ml | 100U/ml & 100ug/ml | 100U/ml & 100ug/ml | - |
| Recovery Cell culture Freezing Medium | - | - | - | 100% |

1.2. Thawing method

1.2.1	Place 9 mL of pre-warmed growth medium into a 15 mL of conical tub.

1.2.2	Remove the vial of cells to be thawed from liquid nitrogen and thaw rapidly by placing at 37°C in a water bath with gentle agitation for 1-2 minutes.

1.2.3	Decontaminate the vial by wiping with 70% ethanol before opening in a biological safety cabinet.

1.2.4	Transfer the vial contents drop-wise into 9 mL of growth medium in a sterile 15-mL conical tub.

1.2.5	Centrifuge cells at 900 rpm for 4 minutes and resuspend in thaw medium.

1.2.6	Transfer contents to the T75 tissue culture flask containing Growth Medium and place flask in a humidified 37°C/5% CO2 incubator.

1.3. Propagation method

1.3.1	Aspirate medium, rinse once in DPBS, add 0.05% Trypsin/EDTA and swirl to coat the cell evenly.

1.3.2	Add an equal volume of Growth Medium to inactivate Trypsin after 2-3 minutes incubation at 37°C.

1.3.3	Centrifuge cells at 900 rpm for 4 minutes and resuspend in growth medium.

1.3.4	Cell should be passage or fed at least twice a week.

2. Assay Protocol

2.1 Harvest and resuspend cells in culture/assay medium

2.2 Dispense 1500 cells/4 uL/well (for agonist mode) into 1536-well tissue treated black/clear bottom plates using a Multi-drop dispenser.

2.3 After the cells were incubated at 37°C overnight, add 4 µL of Loading Dye to each well.

2.4 Incubate the plates for 1 h at 37°C.

2.5 Incubate the plates for 15 min at room temperature.

2.6 Add 23 nL of compounds dissolved in DMSO, positive controls or DMSO by a PinTool.

2.7 Read the fluorescence intensity in FDSS 7000EX kinetic plate reader with a filter set of Ex/Em=480/540 for 5 min at 1 sec intervals

3. Assay Performance

| HEK293 WT (ATP; Agonist control) | Screening Agonist (Mean ± SD) |
| --- | --- |
| EC50 | 92.16 ± 33.9 μM (n = 387) |
| S/B | 14.13 ± 2.81 |
| CV (%) ⃰ | 9.576 ± 1.75 (n = 392) |
| Z’ | 0.58 ± 0.08 |