Protocol of KS_ARE HaCaT human keratinocytes Cell-based Assay for High-throughput Screening (Agonist mode)

| DOCUMENT: |  | KS_ARE_TOX21_SLP_Version1.0 |
| --- | --- | --- |
| TITLE: |  | Protocol of KS_ARE HaCaT human keratinocytes Cell-based Assay for High-throughput Screening |

ASSAY RFERENCES:

| Assay Target | Cell Lines | Species | Tissue of Origin | Assay Readout | Assay Provider | Toxicity Pathway |
| --- | --- | --- | --- | --- | --- | --- |
| Nrf2/ARE | HaCAT | Human | Skin keratinocytes cells | Luciferase reporter | Givaudan, Switzerland | Stress response |

QUALITY CONTROL PRECAUTIONS:

1. The cells should not be grown more than 80-85% confluence

2. The cell performance is affected if they are too confluent

3. Do not leave cells in Trypsin for more than 5 min at RT

4. Spin plate for 5 seconds after One-Glo added

MATERIALS and INSTRUMENTS:

| Supplies/Medium/Reagent | Manufacturer | Vender/Catalog Number |
| --- | --- | --- |
| DMEM+Glutamax | Gibco | 10567-014 |
| FBS | HyClone | SH30071.03, heat inactivated |
| Geneticin (G418) | Gibco | 10131-035 |
| Pen-Strep | Invitrogen | 15140 |
| 0.05% Trypsin-EDTA | Invitrogen | 25300-054 |
| tert-Butylhydroquinone | Sigma | 112941 |
| Multidrop | -Thermofisher | - |
| BiorapTR | Beckman Coulter | - |
| CellTiter- Fluor | Promega | G6080/1/2 |
| Viewlux Plate Reader | Perkin Elmer | - |
| One-Glo | Promega | E606C/E6051 |

PROCEDURE:

1. Cell handling:

1.1. Media Required:

| Component | Growth Medium | Assay Medium | Thaw Medium | Freezing Medium |
| --- | --- | --- | --- | --- |
| Recovery Cell Freezing Medium | - | - | - | 100% |
| DMEM+Glutamax | 90% | 98% | 90% | - |
| FBS | 10% | 1% | 10% | - |
| Penn-strep | 1% | 1% |  | - |
| G418 | 500μg/ml | - | - | - |

1.2. Thawing method

1.2.1 -Place 14 ml of pre-warmed thaw medium into a 15 ml conical tube

1.2.2 -Remove the vial of cells to be thawed from liquid nitrogen and thaw rapidly by placing at 37°C in a water bath with gentle agitation for 1-2 minutes. Do not submerge vial in water.

1.2.3 -Mix the entire content of the vial to 14 ml of pre-warmed medium and centrifuge to remove DMSO

1.2.4 -Discard the supernatant and transfer the precipitated cells to T225 flask using 45 ml thawing medium

1.3. Propagation method

1.3.1 -Detach the cells from the flask using 0.05% Trypsin-EDTA

1.3.2 -The cells are re-seeded in T-225 flask at 2.5 million

2. Assay Protocol

2.1 -Spin down the cells after rinsing the cells with DPBS and trypsinizing

2.2 –Re-suspend the pellet with assay medium followed by filtering through cell strainer and adjust to the required cell density

2.3 -Plate the cells in white-solid bottom 1536 well plate at 1500/well/5µl for agonist mode through 8 tip Multidrop plate dispenser

2.4 -Incubate for 6hrs at 37ºC / 95% Humidity / 5% CO2

2.5 -Transfer 23nL of compounds from the library collection and positive control to the assay plates through pintool

2.6 -Incubate for 23hrs at 37ºC / 95% Humidity / 5% CO2

2.7 -Add 1µl of CellTiter- Fluor using a single tip plate dispenser (Bioraptr)

2.8 -Incubate at 37ºC / 95% Humidity / 5% CO2 for 1hr

2.9 -Read the fluorescence intensity through Viewlux plate reader using protocol optimized for this cell type

2.10 -Add 4µl of One-Glo, spin plate for 5sec and incubate at room temperature for 30min

2.11 -Read luminescence intensity through ViewLux plate reader using protocol optimized for this cell type

3. Assay Performance

3.1 Performance of online validation:

| KS-ARE Agonist (tBHQ) | Online Validation (Mean ± SD) | Online Validation Viability (Mean ± SD) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EC50 | 85.05 ± 15.63 nM (n =27) | N/A |  |  |  |  |  |
| S/B | 3.27 ± 0.17 | 8.89 ± 0.68 |  |  |  |  |  |
| CV (%) | 7.98 ± 1.19 (n = 24) | 3.33 ± 1.33 (n = 24) |  |  |  |  |  |
| Z’ | 0.41 ± 0.12 | 0.87 ± 0.02 |  |  |  |  |  |

* CV values shown represent average of all assay plates excluding the top concentration plates.

3.2 Performance of online screening:

| Assay | Active match | Inactive match | Inconclusive | Mismatch | AC50 fold change | Score |
| --- | --- | --- | --- | --- | --- | --- |
| tox21-ks-are-p1 | 11.60% | 75.74% | 12.30% | 0.35% | 1.40 | 85.93 |
| tox21-ks-are-p1_viability | 9.32% | 84.68% | 5.89% | 0.11% | 1.28 | 97.21 |