Protocol of P450-Glo™ CYP2C19 Assay for High-throughput Screening

| DOCUMENT: |  | P450-Glo™ CYP2C19_TOX21_SLP_Version1.0 |
| --- | --- | --- |
| TITLE: |  | Protocol of P450-Glo™ CYP2C19 Assay for High-throughput Screening |

ASSAY RFERENCES:

| Assay Target | Cell Lines | Species | Tissue of Origin | Assay Readout | Assay Provider | Toxicity Pathway |
| --- | --- | --- | --- | --- | --- | --- |
| CYP2C19 | N/A | Human | Enzyme | Luminescence | Promega Corporation | Cell-free cytochrome P450 |

QUALITY CONTROL PRECAUTIONS:

- Avoid repeated freeze-thaw cycles of substrate and enzyme.

- Dispense the unused Luciferin-H EGE and CYP2C19 membranes into single-use aliquots and store at -70°C.

- Keep the mixtures on ice during assay run.

MATERIALS and INSTRUMENTS:

| Supplies/Medium/Reagent | Manufacturer | Vender/Catalog Number |
| --- | --- | --- |
| -P450-Glo™ CYP2C19 Screening System | -Promega | -Promega/V9880 |
| -Ketoconazole (Positive control compound) | -Sigma Aldrich | -Sigma Aldrich/K1003 |
| -1536-well medium binding white solid plates | -Greiner Bio-one | -Greiner Bio-one/789175-F |
| -BioRAPTR FRD | -Beckman Coulter | -Beckman Coulter |
| -Pintool station | -Wako | -Wako |
| -ViewLux plate reader | -Perkin Elmer | -Perkin Elmer |

PROCEDURE:

- Enzyme-substrate mixture:

- Luciferin-Free water = 21.53uL

- Potassium phosphate buffer, 1M = 2.5uL

- 10mM Luciferin-H EGE = 0.05ul (10mM Luciferin-H EGE solution was prepared by dissolving 123ug of Luciferin-H EGE in 40ul of acetonitrile and mix vigorously)

- CYP2C19 membranes = 0.25ul

- 0.2% BSA (7.5%BSA stock) = 0.67uL

- Final volume = 25uL

- NADPH regeneration solution:

- Luciferin-Free water = 22.0uL

- Solution A = 2.5uL

- Solution B = 0.5uL

- Final Volume = 25.0uL

- Assay Protocol

  - Two uL of enzyme-substrate mixture was dispensed dispenser in 1536-well medium-binding white solid plates using BioRAPTR dispenser.

  - The positive control and test compounds were transferred at 23nL to 1-4 and 5-48 columns of the assay plates respectively using Pintool station.

Positive control plate format: Column 1: two-fold sixteen-point titration starting at 20mM (final = 115uM) duplicates; Columns 2 & 3 (top halves): 20mM (final = 115uM) and 10mM (57.5uM) Ketoconazole respectively; Columns 2 & 3 (bottom halves) and 4: DMSO.

Control used is Ketoconazole [20mM made in DMSO (final = 115uM)]

  - The assay plates were incubated for 10min at room temperature.

  - The reaction was initiated through the addition of 2uL NADPH regeneration solution using BioRAPTR dispenser.

  - The assay plates were incubated for 60min at 37°C to allow the reaction to continue before it was stopped with a detection reagent.

  - Four uL of detection reagent was added using BioRAPTR dispenser.

  - The assay plates were incubated for 20min at 37°C.

  - The luminescence intensity was quantified using ViewLux plate reader (Exposure time: 10sec).

ASSAY PERFORMANCE:

| Online Validation | P450-Glo™ CYP2C19 assay |
| --- | --- |
| AC50 (uM) | 10.91 ± 1.50 (n = 27) |
| CV ⃰ | 6.27 ± 0.99 (n = 27) |
| S/B | 12.13 ± 2.03 |
| Z | 0.73 ± 0.04 |

⃰ CV values shown represent average of DMSO column from each plate.