Protocol of P53-BLA HCT-116 Cell-based Assay for High-throughput Screening

| DOCUMENT: |  | P53-BLA_TOX21_SLP_Version1.0 |
| --- | --- | --- |
| TITLE: |  | Protocol of P53-BLA HCT-116 Cell-based Assay for High-throughput Screening |

ASSAY RFERENCES:

| Assay Target | Cell Lines | Species | Tissue of Origin | Assay Readout | Assay Provider | Toxicity Pathway |
| --- | --- | --- | --- | --- | --- | --- |
| P53 | HCT-116 | Human | Colon carcinoma | Beta-lactamase reporter | Invitrogen | Stress response |

QUALITY CONTROL PRECAUTIONS:

1. Handle the 1536-well, black-wall, clear-bottom assay plate by the sides; do not touch the clear bottom of the assay plate.

MATERIALS and INSTRUMENTS:

| Supplies/Medium/Reagent | Manufacturer | Vender/Catalog Number |
| --- | --- | --- |
| McCoy's 5A Medium | Invitrogen | Invitrogen, 16600-108 |
| Opti-MEN Reduced Serum Medium | Invitrogen | Invitrogen, 11058-021 |
| Fetal bovine serum, dialyzed | Invitrogen | Invitrogen, 26400 |
| Nonessential amino acids (NEAA) | Invitrogen | Invitrogen, 11140 |
| Penicillin/Streptomycin (antibiotic) | Invitrogen | Invitrogen, 15140 |
| DPBS | Invitrogen | Invitrogen, 14190 |
| Sodium pyruvate | Invitrogen | Invitrogen, 11360 |
| 0.25% Trypsin/EDTA | Invitrogen | Invitrogen, 25300 |
| Blasticidin (antibiotic) | Invitrogen | Invitrogen, R210 |
| DMSO | AMRESCO | KD Medical, RGE-3070 |
| Recovery Cell Culture | Invitrogen | Invitrogen, 12648 |
| LiveBLAzer FRET B/G Loading Kit: | Invitrogen | Invitrogen , K1030 |
| Solution D | Invitrogen | Invitrogen, K1157 |
| Mitomycin C | Calbiochem | Calbiochem, 475820 |
| Nutlin-3 | Calbiochem | Calbiochem, 444143 |
| Black-wall, clear-bottom, 1536-well assay plates | Greiner Bio-One | Greiner, 789092-F |
| PinTool | Kalypsys | - |
| BioRAPTR, Microfluidic Workstation | Beckmen | - |
| EnVision plate reader | Perkin Elmer | - |
| Centrifuge | Sorvall legend XTR | Thermo Fisher Science, 75004520 |

PROCEDURE:

1. Cell handling:

1.1. Media Required:

| Component | Growth Medium | Assay Medium | Thaw Medium | Freezing Medium |
| --- | --- | --- | --- | --- |
| McCoy's 5A medium | 90% | - | 90% | - |
| Opti-MEN | - | 99.5% | - | - |
| Dialyzed FBS | 10% | 0.5% | 10% | - |
| NEAA | - | 0.1 mM | - | - |
| Sodium pyruvate | - | 1mM | - | - |
| Penicillin-Streptomycin | 100 U/mL-100 µg/mL | 100 U/mL-100 µg/mL | 100 U/mL-100 µg/mL | - |
| Blasticidin (antibiotic) | 5 µg/mL | - | - | - |
| Recovery Cell Culture | - | - | - | 100% |

1.2. Thawing method

1.2.1 Place 14 mL of pre-warmed thaw medium into a 15 mL of conical tub.

1.2.2 Remove the vial of cells to be thawed from liquid nitrogen and thaw rapidly by placing at 37°C in a water bath with gentle agitation for 1-2 minutes.

1.2.3 Decontaminate the vial by wiping with 70% ethanol before opening in a biological safety cabinet.

1.2.4 Transfer the vial contents drop-wise into 14 mL of thaw medium in a sterile 15-mL conical tub.

1.2.5 Centrifuge cells at 900 rpm for 4 minutes and resuspend in thaw medium.

1.2.6 Transfer contents to the T75 tissue culture flask containing Thaw Medium and place flask in a humidified 37°C/5% CO2 incubator.

1.2.7 Switch to growth medium at first passage.

1.3. Propagation method

1.3.1 Aspirate medium, rinse once in DPBS, add 0.25% Trypsin/EDTA (3 mL for a T75 flask and 5 mL for a T175 flask and 7.5 mL for T225 flask) and swirl to coat the cell evenly.

1.3.2 Add an equal volume of Growth Medium to inactivate Trypsin after 2-3 minutes incubation at 37°C.

1.3.3 Centrifuge cells at 900 rpm for 4 minutes and resuspend in growth medium.

1.3.4 Cell should be passage or fed at least twice a week.

2. Assay Protocol

2.1 Harvest cells from culture in growth medium and resuspend in assay medium.

2.2 Dispense 4000 cells/7 µL/well into 1536-well black/clear bottom plates using a Multidrop dispenser.

2.3 After the cells were incubated at 37°C and 5% CO2 for 5 hours, 23 nL of compounds dissolved in DMSO, positive controls or DMSO were transferred to the assay plate by a PinTool resulting in a 217-fold dilution.

2.4 Incubate the plates for 16 hours at 37°C, 5% CO2.

2.5 Add 1 µL of 8X LiveBLAzer FRET B/G (CCF4-AM) Substrate Mixture to each well using a BioRAPTR dispenser and incubate the plates at room temperature for 2 hours.

2.6 Measure fluorescence intensity at 460 and 530 nm emission and 405 nm excitation by an Envision detector. Data is expressed as the ratio of 460nm/530nm emissions.

2.7 Add 3 uL of Cell-titer Glo to each well using BioRAPTR and incubate the plates at room temperature for 30 min

2.8 Measure luminescence intensity by a VewLux plate reader.

3. Assay performance: