Protocol of PPAR-delta-BLA HEK 293T Cell-based Assay for High-throughput Screening

| DOCUMENT: |  | PPAR-delta-BLA_TOX21_SLP_Version1.0 |
| --- | --- | --- |
| TITLE: |  | Protocol of PPAR-delta-BLA HEK 293T Cell-based Assay for High-throughput Screening |

ASSAY RFERENCES:

| Assay Target | Cell Lines | Species | Tissue of Origin | Assay Readout | Assay Provider | Toxicity Pathway |
| --- | --- | --- | --- | --- | --- | --- |
| PPARdelta: LBD (Recombinant) | HEK 293T | Human | Embryonic kidney cells | Beta-lactamase reporter | Invitrogen | NR signaling |

QUALITY CONTROL PRECAUTIONS:

1. -The cells should not be grown more than 80-85% confluence

2. -The cell performance is affected is they are more confluent

3. - Do not leave cells in Trypsin for more than 5 min at RT

4. -handle 1536 well plate black clear bottom plates carefully by sides

MATERIALS and INSTRUMENTS:

| Supplies/Medium/Reagent | Manufacturer | Vender/Catalog Number |
| --- | --- | --- |
| -DMEM+Glutamax | -Invitrogen | -10569 |
| -Phenol Red free DMEM | -Invitrogen | -21063 |
| -Dialyzed FBS | -Invitrogen | -26400 |
| -Charcoal Stripped FBS | -Invitrogen | -12676 |
| -Sodium Pyruvate | -Invitrogen | -11360 |
| -Pen-Strep | -Invitrogen | -15140 |
| -NEAA | -Invitrogen | -11140 |
| -HEPES | -Invitrogen | -15630 |
| -HygromycinB | -Invitrogen | -10687 |
| -Zeocin | -Invitrogen | -R25001 |
| -Multidrop | -Thermofisher | - |
| -BiorapTR | -Beckman Coulter | - |
| -Envision Plate Reader | -Perkin Elmer | - |
| -LiveBLAzer B/G FRET substrate | -Invitrogen | -K1030 |

PROCEDURE:

1. Cell handling:

1.1. Media Required:

| Component | Growth Medium | Assay Medium | Thaw Medium | Freezing Medium |
| --- | --- | --- | --- | --- |
| -Recovery Cell Freezing Medium | - | - | - | -100% |
| -DMEM+Glutamax | -90% | - | -90% | undefined |
| -Phenol Red free DMEM | - | -98% | - | - |
| -Dialyzed FBS | -10% | - | -10% | undefined |
| -Charcoal-Stripped FBS | - | -2% | - | - |
| -Penn-strep | -1% | -1% | -1% | - |
| -Sodium Pyruvate | - | - | -1 mM | - |
| -HEPES | -25 mM | - | - | - |
| -NEAA | -0.1 mM | - | -0.1 mM | - |
| -Hygromycin | - 80 ug/ml | - | - | - |
| -Zeocin | -100 ug/ml | - | - | - |

1.2. Thawing method

1.2.1 -Place 14 mL of pre-warmed thaw medium into a 15 ml conical tube

1.2.2 -Remove the vial of cells to be thawed from liquid nitrogen and thaw rapidly by placing at 37°C in a water bath with gentle agitation for 1-2 minutes. Do not submerge vial in water.

1.2.3 -Mix the entire content of the vial to 14 ml of pre-warmed medium and centrifuge to remove DMSO

1.2.4 -Discard the supernatant and transfer the precipitated cells to T175 flask using 30 ml thawing medium

1.3. Propagation method

1.3.1 -Detach the cells from the flask using TrypLExpress

1.3.2 -The cells are re-seeded in T-175 flask at 2.5-4 million

2. Assay Protocol

2.1 -Spin down the cells after rinsing the cells with DPBS and trypsinizing

2.2 -Resuspend the pellet with assay medium followed by filtering through cell strainer and adjust the required cell density

2.3 -Plate the cells in black-clear bottom 1536 well plate at 3000/well/6uL for agonist mode and 3000cells/well/5 ul for antagonist mode through 8 tip Multidrop plate dispenser

2.4 -Incubate for 5 hrs at 37ºC / 99% Humidity / 5% CO2

2.5 -Transfer 23nL of compounds from the library collection and positive control to the assay plates through pintool

2.6 -Add 1 uL of buffer and 1uL of Agonist concentration to respective columns as per plate map for antagonist mode

2.7 -Incubate for 17 hrs at 37ºC / 99% Humidity / 5% CO2

2.8 -Add 1uL of CCF4 (FRET Substarte) dye using a single tip plate dispenser (Bioraptr)

2.9 -Incubate at room temperature for 2 hrs in dark

2.10 -Read the fluorescence intensity through Envision plate reader using Beta-Lactamaze protocol optimized for this cell type

2.11 -Add 3 uL of Cell Titer Glo and Incubate at room temperature for 0.5 hrs in dark for both agonist and antagonist mode

2.12 -Read on ViewLux protocol optimized for this cell type for both agonist and antagonist mode

3. Assay Performance

| PPARd-bla Antagonist (MK886) | Online Validation (Mean ± SD) | Online Validation Viability (Mean ± SD) |
| --- | --- | --- |
| IC50 | 17.96 ± 3.59 μM (n =27) | N/A |
| S/B | 4.00 ± 0.20 | 99.30 ± 15.75 |
| CV (%) | 5.75 ±0.48* (n = 20) | 7.30 ± 0.44 (n = 20) |
| Z’ | 0.75 ± 0.05 | 0.83 ±0.03 |

* CV values shown represent average of DMSO plates and low concentration plates only.