Protocol of HEK293-TRHR Cell-based Assay for High-throughput Screening

| DOCUMENT: |  | HEK293-TRHR_TOX21_SLP_Version1.0 |
| --- | --- | --- |
| TITLE: |  | Protocol of HEK293-TRHR Cell-based Assay for High-throughput Screening |

ASSAY RFERENCES:

| Assay Target | Cell Lines | Species | Tissue of Origin | Assay Readout | Assay Provider | Toxicity Pathway |
| --- | --- | --- | --- | --- | --- | --- |
| TRHR | Hek293- TRHR | Human | Embryonic kidney | Fluorescence | Codex | Gs-coupled TRHR signaling |

QUALITY CONTROL PRECAUTIONS:

1. The cells should not be grown more than 80-85% confluence

2. Do not leave cells in Trypsin for more than 5 min at RT

MATERIALS and INSTRUMENTS:

| Supplies/Medium/Reagent | Manufacturer | Vender/Catalog Number |
| --- | --- | --- |
| DMEM, high-glucose | Invitrogen | 11995 |
| FBS | Invitrogen | 26140 |
| Penicillin/Streptomycin (antibiotic) | Invitrogen | 15140 |
| Puromycin | Invitrogen | A11138 |
| Recovery Cell Culture Freezing Medium | Invitrogen | 12648 |
| Cal-520 calcium assay kit | AAT Bioquest | 36400 |
| TRH | Sigma | P1319 |
| FDSS 7000EX kinetic plate reader | FDSS 7000EX | - |
| Multidrop | Thermofisher | - |
| BioRAPTR | Beckman Coulter | - |

PROCEDURE:

1. Cell handling:

1.1. Media Required:

| Component | Growth Medium | Assay Medium | Thaw Medium | Freezing Medium |
| --- | --- | --- | --- | --- |
| DMEM, high glucose | 90% | 90% | 90% | - |
| FBS | 10% | 10% | 10% | - |
| Penicillin/Streptomycin (antibiotic) | 1% | 1% | 1% | - |
| Puromycin | 1ug/mL | 1ug/mL | - | - |
| Recovery Cell Culture Freezing Medium | - | - | - | 100% |

1.2. Thawing method

1.2.1 Place 14 mL of pre-warmed thaw medium into a 15 mL conical tube

1.2.2 Remove the vial of cells to be thawed from liquid nitrogen and thaw rapidly by placing at 37°C in a water bath

1.2.3 Mix the entire content of the vial to 14 ml of pre-warmed medium and centrifuge to remove DMSO

1.2.4 Discard the supernatant and reconstitute the pellet using pre-warmed thaw medium

1.2.5 Transfer the cells to T75 flask

1.3. Propagation method

1.3.1 Detach the cells from the flask using Trypsin-EDTA (0.05%)

1.3.2 The cells in growth medium are re-seeded in T225 flask

2. Assay Protocol

2.1 Harvest cells and re-suspend in assay medium and adjust the required cell density

2.2 Dispense 1500 cells/4 µL/well into 1536 well tissue treated black-clear-bottom plates using a Multidrop dispenser

2.3 Incubate the plates at a 37ºC, 5% CO2 incubator for 18 h

2.4 Add 4 µL of Loading Buffer

2.5 Incubate at 37ºC, 5% CO2 for 2 h

2.6 Incubate at RT for 15 min

2.7 Transfer 23nL of compounds and positive control to the assay plate by a pin tool.

2.8 Read the fluorescence intensity in FDSS 7000EX kinetic plate reader with a filter set of Ex/Em=480/540 for 3 min at 1 sec intervals

2.9 For antagonist mode, add 1 µL of TRH at 0.1 nM (final) to each well except 16 wells in column 4 after 5 min incubation at RT.

3.0 Read the fluorescence intensity in FDSS 7000EX kinetic plate reader with a filter set of Ex/Em=480/540 for 3 min at 1 sec intervals

3. Assay Performance

| TRHR-agonist (TSH) | Online Validation (Mean ± SD) |
| --- | --- |
| AC50 (nM) | 0.05±0.04 |
| S/B | 48.15 ± 12.93 |
| CV (%) | 25.90 ± 8.17* |
| Z’ | 0.79 ± 0.06 |
| N | 27 |

| TRHR-antagonist (N/A) | Online Validation (Mean ± SD) |
| --- | --- |
| AC50 (nM) | N/A |
| S/B | 62.58 ± 11.05 |
| CV (%) | 6.70 ± 1.32* |
| Z’ | 0.81 ± 0.05 |
| N | 27 |

*CV values shown represent average of DMSO plates and low concentration plates only (n=18).